Module 2

Practical 10

Operations on BCD numbers.

- Add two 4 digit BCD numbers in HL and DE register pairs and store result

in memory locations, 2300H and 2301H. Ignore carry after 16 bit.

Code:

LXI H,0204H

LXI D,0203H

MOV A,L

ADD E

DAA

MOV L,A

MOV A,H

ADC D

DAA

MOV H,A

SHLD 0200H

HLT

OUTPUT:

| Before execution | After execution |
| --- | --- |
| 0200H=XX | 0200H=07H |
| 0201H=XX | 0201H=04H |

- Subtract the BCD number stored in E register from the number stored in

the D register.

Code:

MVI D,15H

MVI E,12H

MVI A,99H

SUB E

INR A

ADD D

DAA

STA 0300H

HLT

OUTPUT:

| Before execution | After execution |
| --- | --- |
| 0300H=XX | 0300H=03H |

- Write an assembly language program to multiply 2 BCD numbers.

Code:

LDA 0200H

MOV C,A

SUB A

LXI H,0201H

Back: ADD M

DAA

XYZ: DCR C

JNZ Back

STA 0300H

HLT

OUTPUT:

| Before execution | After execution |
| --- | --- |
| 0200H=XX | 0200H=03H |
| 0201H=XX | 0201H=02H |
| 0300H=XX | 0300H=06H |