SUB- ASSEMBLY LANGUAGE PROGRAMMING.

CLASS: FY.BSC.IT                                                                                                                                                                                                                DOP-14/12/24 and 17/12/24

DOA-

PRACTICAL -2

2. Simple assembly language programs.

a. Subtract the contents of memory location 4001H from the memory

location 2000H and place the result in memory location 4002H.

SOLN

LDA 4001H

MOV B,A

LDA 2000H

SUB B

STA 4002H

HLT

OUTPUT:

| Before Execution | After Execution |
| --- | --- |
| 2000H=05H | 2000H=05H |
| 4001H= 02H | 4001H= 02H |
| 4002H=XX | 4002H=03H |

RESULT:

b. Subtract two 8-bit numbers.

SOLN:

MVI A,09H

MVI B,05H

SUB B

STA 2000H

HLT

OUTPUT:

| Before Execution | After Execution |
| --- | --- |
| A=XX | A=4H |
| B=XX | B=5H |
| 2000H=XX | 2000H=4H |

RESULT:

c. Add the 16-bit number in memory locations 4000H and 4001H to the 16-

bit number in memory locations 4002H and 4003H. The most significant

eight bits of the two numbers to be added are in memory locations 4001H

and 4003H. Store the result in memory locations 4004H and 4005H with

the most significant byte in memory location 4005H.

NOTE:

EXPLANATION:

- LHLD 16BIT ADDRESS:

- SHLD 16BIT ADDRESS:

- XCHG:

The XCHG instruction in the 8085 microprocessor exchanges the contents of the HL and DE register pairs.

SOLN:

LHLD 4000H

XCHG

LHLD 4002H

DAD D

SHLD 4004H

HLT

OUTPUT:

| Before Execution | After Execution |
| --- | --- |
| 4000H=2H | 4000H=2 |
| 4001H=3H | 4001H=3 |
| 4002H=4H | 4002H=4 |
| 4003H=5H | 4003H=5 |
| 4004H=XX | 4004H=6 |
| 4005H=XX | 4005H=8 |

RESULT:

d. Add the contents of memory locations 4000H and 4001H and place the

result in the memory locations 4002H and 4003H.

SOLN:

LHLD 4000H

MVI B,00H

MOV A,H

ADD L

JNC ABC

INR B

ABC: STA 4002H

MOV A,B

STA 4003H

HLT

OUTPUT:

| Before Execution | After Execution |
| --- | --- |
| 4000H=2H | 4000H=2H |
| 4001H=5H | 4001H=5H |
| 4002H=XX | 4002H=7H |
| 4003H=XX | 4003H=0H |

RESULT:

f. Find the l's complement of the number stored at memory location 4400H

and store the complemented number at memory location 4300H.

SOLN:

LDA 4400H

CMA

STA 4300H

HLT

OUTPUT:

| Before Execution | After Execution |
| --- | --- |
| 4400H=(15)10=(0F)H | 4400H=(15)10=(OF)H |
| 4300= XX | 4300H=(240)10=(0F)H |

RESULT-

g. Find the 2's complement of the number stored at memory location 4200H

and store the complemented number at memory location 4300H

SOLN:

LDA 0100H

CMA

ADI 01H

STA 0200H

HLT

OUTPUT:

| Before Execution | After Execution |
| --- | --- |
| 0100H=(20)10=14H | 0100H=(20)10=14H |
| 0200H= XXH | 0200H=(236)10=ECH |

RESULT=