Practical  5

5. Multiple memory locations.

a. Calculate the sum of series of numbers. The length of the series is in memory location 200H and the series begins from memory location 201H. i) Consider the sum to be 8 bit number. So, ignore carries. Store the sum at memory location 300H. ii) Consider the sum to be 16 bit number. Store the sum at memory locations 300H and 301H

Code:

i)

LDA  0200H

MOV C,A

SUB A

LXI H,0201H

Back : ADD M

INX H

DCR C

JNZ Back

STA 0300H

HLT

OUTPUT:

| Before Execution | After Execution |
| --- | --- |
| 0200H=XX | 0200H=05H |
| 0201H=XX | 0201H=01H |
| 0202H=XX | 0202H=02H |
| 0203H=XX | 0203H=03H |
| 0204H=XX | 0204H=04H |
| 0205H=XX | 0205H=05H |
| 0300H=XX | 0300H=0FH |

b. Multiply two 8-bit numbers stored in memory locations 200H and 201H by repetitive addition and store the result in memory locations 300H and 301H.

Code:

LDA 0200H

MVI B,00H

MOV C,A

SUB A

LXI H,0201H

Back: ADD M

JNC XYZ

INR B

XYZ : DCR C

JNZ Back

STA 0300H

MOV A,B

STA 0301H

HLT

OUTPUT:

| Before Execution | After Execution |
| --- | --- |
| 0200H=XX | 0200H=01H |
| 0201H=XX | 0201H=02H |
| 0300H=XX | 0300H=02H |
| 0301H=XX | 0301H=00H |

d.Find the number of negative elements (most significant bit 1) in a block of data. The length of the block is in memory location 200H and the block itself begins in memory location 201H. Store the number of negative elements in memory location 300H.

Code:

LDA 0200H

MOV C,A

MVI B,00H

LXI H,0201H

Back : MOV A,M

ANI 80H

JZ Skip

INR B

Skip : INX H

DCR C

JNZ Back

MOV A,B

STA 0300H

HLT

OUTPUT:

| Before Execution | After Execution |
| --- | --- |
| 0200H=XX | 0200H=05H |
| 0201H=XX | 0201H=01H |
| 0202H=XX | 0202H=82H |
| 0203H=XX | 0203H=03H |
| 0204H=XX | 0204H=84H |
| 0205H=XX | 0205H=05H |
| 0300H=XX | 0300H=02H |

e. Find the largest number in a block of data. The length of the block is in memory location 200H and the block itself starts from memory location 201H. Store the maximum number in memory location 300H. Assume that the numbers in the block are all 8 bit unsigned binary numbers.

Code:

LDA 0200H

MOV C,A

SUB A

LXI H,0201H

Back : CMP M

JNC Skip

MOV A,M

Skip : INX H

DCR C

JNZ Back

STA 0300H

HLT

OUTPUT:

| Before Execution | After Execution |
| --- | --- |
| 0200H=XX | 0200H=05H |
| 0201H=XX | 0201H=01H |
| 0202H=XX | 0202H=02H |
| 0203H=XX | 0203H=05H |
| 0204H=XX | 0204H=03H |
| 0205H=XX | 0205H=04H |
| 0300H=XX | 0300H=05H |