Module 2

Practical 1(6)

Calculations with respect to memory locations

b) Calculate the sum of series of even numbers from the list of numbers.

The length of the list is in memory location 200H and the series itself

begins from memory location 201H. Assume the sum to be 8 bit number

so you can ignore carries and store the sum at memory location 300H

Code:

LDA 0200H

MOV C,A

MVI B,00H

LXI H,0201H

Back: MOV A,M

ANI 01H

JNZ Skip

MOV A,M

ADD B

MOV B,A

Skip: INX H

DCR C

JNZ Back

MOV A,B

STA 0300H

HLT

OUTPUT:-

| BEFORE EXECUTION | AFTER EXECUTION |
| --- | --- |
| 0200H=05H | 0200H=05H |
| 0201H=07H | 0201H=07H |
| 0202H=02H | 0202H=02H |
| 0203H=03H | 0203H=03H |
| 0204H=04H | 0204H=04H |
| 0205H=05H | 0205H=05H |
| 0300H=XX | 0300H=06H |

d) Find the square of the given numbers from memory location 400H and

store the result from memory location 600H

Code:

LXI H,0500H :Initialize lookup table pointer

LXI D,0400H:Initialize source pointer

LXI B,0600H:Initialize destination pointer

Back: LDAX D: Get the number

MOV C,A: Move the number to C

MOV A,M: Get the square

STAX B: Store the result at the destination memory location

INX D: Increment source memory pointer

INX B: Increment destination memory pointer

INX H : Increment look up pointer pointer

MOV A,C: Move the number from  C to A

CPI 05H: check for the last number

JNZ Back: If not repeat

HLT: Terminate the program

OUTPUT:-

| BEFORE EXECUTION | AFTER EXECUTION |
| --- | --- |
| 0400H=0H | 0400H=0H |
| 0401H=01H | 0401H=01H |
| 0402H=02H | 0402H=02H |
| 0403H=03H | 0403H=03H |
| 0404H=04H | 0404H=04H |
| 0500H=0H | 0500H=0H |
| 0501H=01H | 0501H=01H |
| 0502H=04H | 0502H=04H |
| 0503H=09H | 0503H=09H |
| 0504H=16H | 0504H=16H |
| 0600H=XX | 0600H=0H |
| 0601H=XX | 0601H=01H |
| 0602H=XX | 0602H=04H |
| 0603H=XX | 0603H=09H |
| 0604H=XX | 0604H=16H |

f) Two decimal numbers six digits each, are stored in BCD package form. Each number occupies a sequence of byte in the memory. The starting address of first number is 100H .Write an assembly language program that adds these two numbers and stores the sum in the same format starting from memory location 300H.

code:

LXI H,0100H

LXI D,0106H

LXI B,0300H

Back: LDAX D

ADD M

DAA

STAX B

INX H

INX D

INX B

MOV A,L

CPI 06H

JNZ Back

HLT

OUTPUT:-

| BEFORE EXECUTION | AFTER EXECTION |
| --- | --- |
| 0100H=02H | 0100H=02H |
| 0101H=01H | 0101H=01H |
| 0102H=02H | 0102H=02H |
| 0103H=01H | 0103H=01H |
| 0104H=02H | 0104H=02H |
| 0105H=01H | 0105H=01H |
| 0106H=02H | 0106H=02H |
| 0107H=01H | 0107H=01H |
| 0108H=02H | 0108H=02H |
| 0109H=01H | 0109H=01H |
| 010AH=02H | 010AH=02H |
| 010BH=01H | 010BH=01H |
| 0300H=XX | 0300H=04H |
| 0301H=XX | 0301H=02H |
| 0302H=XX | 0302H=04H |
| 0303H=XX | 0303H=02H |
| 0304H=XX | 0304H=04H |
| 0305H=XX | 0305H=02H |