Assembly programs on memory locations.

- Write an assembly language program to separate even numbers from the given list of 10 numbers and store them in the another list starting from 500H. Assume starting address of 10 number list is 100H

Code:

LXI H,0100H

LXI D,0500H

MVI C,0AH

Back: MOV A,M

ANI 01H

JNZ Skip

MOV A,M

STAX D

INX D

Skip: INX H

DCR C

JNZ Back

HLT

Output:-

| Before execution | After execution |
| --- | --- |
| 0100=01h | 0100=01h |
| 0101=02h | 0101=02h |
| 0102=03h | 0102=03h |
| 0103=04h | 0103=04h |
| 0104=05h | 0104=05h |
| 0105=06h | 0105=06h |
| 0106=07h | 0106=07h |
| 0107=08h | 0107=08h |
| 0108=09h | 0108=09h |
| 0109=10h | 0109=10h |
| 0500=XX | 0500=02h |
| 0501=XX | 0501=04h |
| 0502=XX | 0502=06h |
| 0503=XX | 0503=08h |
| 0504=XX | 0504=10h |

- A block of data consisting of 10 bytes is stored in memory starting at 300H. This block is to be shifted (relocated) in memory from 350H onwards. Do not shift the block or part of the block anywhere else in the memory.

Code:

MVI D,0AH

LXI H,0300H

LXI B,0350H

ABC : MOV A,M

STAX B

INX H

INX B

DCR D

JNZ ABC

HLT

Output :-

| Before execution | After execution |
| --- | --- |
| 0300=01h | 0300=01h |
| 0301=02h | 0301=02h |
| 0302=03h | 0302=03h |
| 0303=04h | 0303=04h |
| 0304=05h | 0304=05h |
| 0305=06h | 0305=06h |
| 0306=07h | 0306=07h |
| 0307=08h | 0307=08h |
| 0308=09h | 0308=09h |
| 0309=10h | 0309=10h |
| 0350=XX | 0350=01h |
| 0351=XX | 0351=02h |
| 0352=XX | 0352=03h |
| 0353=XX | 0353=04h |
| 0354=XX | 0354=05h |
| 0355=XX | 0355=06h |
| 0356=XX | 0356=07h |
| 0357=XX | 0357=08h |
| 0358=XX | 0358=09h |
| 0359=XX | 0359=10h |

d) Add even parity to a string of 7-bit ASCII characters. The length of the

string is in memory location 200H and the string itself begins in memory

location 201H. Place even parity in the most significant bit of each

character.

Code:

LDA 0200H

MOV B,A

LXI H,0201H

ABC: MOV A,M

ORI 80H

MOV M,A

INX H

DCR B

JNZ ABC

HLT

OUTPUT:-

| Before execution | After execution |
| --- | --- |
| 0200=05h | 0200=05h |
| 0201=03H | 0201=(131)10=83H |
| 0202=01H | 0201=(120)10=81H |
| 0203=06H | 0203=(134)10=86H |
| 0204=04H | 0204=(132)10=84H |
| 0205=07H | 0205=(135)10=87H |

e) A list of 10 numbers is stored in memory, starting at 300H. Find number

of negative, zero and positive numbers from this list and store these

results in memory locations 400H, 401H, and 402H respectively

Code:

LXI H,0300H

MVI C,0AH   ;Iniilize counter

MVI B,00H   ;store Negative nos

MVI E,00H   ;store Zero nos

MVI D,00H   ;store positive nos

Begin: MOV A,M

CPI 00H

JZ ZeroNum

ANI 80H

JNZ NegNum

INR D

JMP Last

ZeroNum: INR E

JMP Last

NegNum: INR B

Last: INX H

DCR C

JNZ Begin

LXI H,400H

MOV M,B

INX H

MOV M,E

INX H

MOV M,D

HLT

OUTPUT:-

| Before execution | After execution |
| --- | --- |
| 0300=00H | 0300=00H |
| 0301=02H | 0301=02H |
| 0302=(135)10=87H | 0302=(135)10=87H |
| 0303=04H | 0303=04H |
| 0304=05H | 0304=05H |
| 0305=06H | 0305=06H |
| 0306=(129)10=81H | 0306=(129)10=81H |
| 0307=08H | 0307=08H |
| 0308=09H | 0308=09H |
| 0309=00H | 0309=00H |
| 0400=XX | 0400=02H |
| 0401=XX | 0401=02H |
| 0402=XX | 0402=06H |

g) Program to calculate the factorial of a number between 0 to 8.

Code:

MVI B,05        ; PUT THE VALUE OF 5 IN THE B REGSITER

MOV C,B         ; COPY B INTO C

DCR B           ; DECREMENT THE B REGISTER

L1: MOV D,B     ; CREATE THE L1 SECTION AND MOVE B INTO D

MVI A,00        ; PUT 0 IN THE ACCUMULATOR

L2: ADD C       ; CREATE THE L2 SECTION AND MOVE ADD C TO THE ACCUMULATOR

DCR D           ; DECREASE D

JNZ L2          ; JUMP BACK TO L2

MOV C,A         ; MOVE A INTO C

DCR B           ; DECREASE B BY 1

JNZ L1          ; JUMP TO L1

HLT

OUTPUT:-

| Before execution | After execution |
| --- | --- |
| A=XX | A=(78)10=120 |