Module 2

Practical 8

String operations in assembly programs.

c) Multiply the 8-bit unsigned number in memory location 200H by the 8-

bit unsigned number in memory location 201H. Store the 8 least

significant bits of the result in memory location 300H and the 8 most

significant bits in memory location 301H

code:

LDA 0200H

MVI B,00H

MOV C,A

SUB A

LXI H,0201H

Back : ADD M

JNC XYZ

INR B

XYZ : DCR C

JNZ Back

STA 0300H

MOV A,B

STA 0301H

HLT

OUTPUT:

| Before execution | After execution |
| --- | --- |
| 0200H=XX | 0200H=03H |
| 0201H=XX | 0201H=02H |
| 0300H=XX | 0300H=06H |
| 0301H=XX | 0301H=00H |

e) DAA instruction is not present. Write a sub routine which will perform the

same task as DAA.

Code:

MVI A,7DH

MOV B,A

ANI 0FH

CPI 0AH

JC UPPER

MOV A,B

ADI 06H

MOV B,A

JMP UPPER1

UPPER: MOV A,B

UPPER1: ANI 0F0H

RRC

RRC

RRC

RRC

CPI 0AH

JC XYZ

MOV A,B

ADI 60H

JMP ABC

XYZ: MOV A,B

ABC: HLT

OUTPUT:

Note:Assume Accumulator has 7DH Data, which is to be converted to Decimal by logic of DAA instruction

| Before execution | After execution |
| --- | --- |
| A=XX | A=83H |