Module 2
Practical 9

- c). Transfer ten bytes of data from one memory to another memory block.

Source memory block starts from memory location 300H where as

destination memory block starts from memory location 350H.

Code:

MVI D,0AH

LXI H,0300H

LXI B,0350H

ABC : MOV A,M

STAX B

INX H

INX B

DCR D

JNZ ABC

HLT

OUTPUT:

| Before execution | After execution |
| --- | --- |
| 0300H=XX | 0300H=00H |
| 0301H=XX | 0301H=01H |
| 0302H=XX | 0302H=02H |
| 0303H=XX | 0303H=03H |
| 0304H=XX | 0304H=04H |
| 0305H=XX | 0305H=05H |
| 0306H=XX | 0306H=06H |
| 0307H=XX | 0307H=07H |
| 0308H=XX | 0308H=08H |
| 0309H=XX | 0309H=09H |
| 0350H=XX | 0350H=00H |
| 0351H=XX | 0351H=01H |
| 0352H=XX | 0352H=02H |
| 0353H=XX | 0353H=03H |
| 0354H=XX | 0354H=04H |
| 0355H=XX | 0355H=05H |
| 0356H=XX | 0356H=06H |
| 0357H=XX | 0357H=07H |
| 0358H=XX | 0358H=08H |
| 0359H=XX | 0359H=09H |

- e). Write a simple program to Split a HEX data into two nibbles and store it

in memory.

Code:

LXI H,0200H

MOV A,M

MOV B,A

ANI 0FH

INX H

MOV M,A

MOV A,B

ANI 0F0H

RRC

RRC

RRC

RRC

INX H

MOV M,A

HLT

OUTPUT:

| Before execution | After execution |
| --- | --- |
| 0200H=XX | 0200H=23H |
| 0201H=XX | 0201H=03H |
| 0202H=XX | 0202H=02H |